Low Energy Bluetooth Service Broadcasting using ESP32 Devkit V1

By Nirvan Tamhane

Table of Contents

- Task Details

- Hardware Components Used

- Tools Used

- Microcontroller Interfacing

- Schematic

- Complete Hardware Assembly

- Firmware

- Project Overview Video

- Issues Faced and Resolution

- Outputs

- nRF Connect Application Interface

- References

Task Details

Services to be Broadcasted:

- Temperature Measurement

- Humidity

Details about the Service to Broadcast Over Bluetooth:

- Service UUID: 00000002-0000-0000-FDFD-FDFDFDFDFDFD

- Characteristics:

  - Temperature Measurement: Standard BLE characteristic - GATT Char UUID: 0x2A1C

  - Humidity: Standard BLE characteristic - GATT Char UUID: 0x2A6F

- Supported Operations: Both characteristics support read and notify.

Hardware Components Used

- Espressif System's ESP32 Devkit V1 Development Board

- DHT11 Temperature and Humidity Sensor

- 10K Ohm Resistor (Pull-up)

- USB to MicroUSB Cable

- Breadboard

- Jumpers

Tools Used

Arduino IDE

Arduino Integrated Development Environment or Arduino Software (IDE) contains a text editor for writing code, a message area, a text console, a toolbar with buttons for common functions, and a series of menus. It connects to the Arduino hardware to upload programs and communicate with them.

- For more details, refer to: Arduino IDE

Fritzing

Fritzing is an open-source hardware initiative that makes electronics accessible as creative material for anyone. We offer a software tool, a community website, and services in the spirit of Processing and Arduino, fostering a creative ecosystem that allows users to document their prototypes, share them with others, teach electronics in a classroom, and layout and manufacture professional PCBs.

- For more details, refer to: Fritzing

Microcontroller Interfacing

- Used OneWire protocol to interface the microcontroller to the DHT11 sensor.

- Used BLE protocol for Connectivity with the nRF Connect Mobile Application.

Schematic

Complete Hardware Assembly

Firmware

Link: Firmware .ino File

Project Overview Video

Link: Project Overview Video

Issues Faced and Resolution

Approach 1:

- Used the callback function to restart advertising within the loop itself.

- Result: Successfully connected and reconnected multiple times with no issues (Check at the end of the Project Overview Video).

ESP32 Not Advertising After Disconnection

Approach 1:

- Started with using custom characteristic UUIDs and tried to send the data as String values (I had worked on this before).

- Result: Successfully received both values Temperature Measurement & Humidity in Strings.

- Comment: This was not asked in the task. It was mandatory to use the default GATT UUIDs.

BLE Data Transmission Problem

Approach 2:

- Set the UUIDs to the required UUIDs (Temperature Measurement = 0x2A1C; Humidity = 0x2A6F) and tried to send the data as String values.

- Result: Error: Invalid Data Syntax @ nRF Application for Temperature Measurement value & Garbage value for Humidity value.

- Comment: This was not asked in the task. Getting the Humidity in % and Temperature Measurement in Celsius was mandatory.

Approach 3:

- Changed the Temperature characteristic UUID to default temperature (Temperature = 0x2AE6) and tried to send the data as unsigned 16-bit int.

- Result: Successfully received both values Temperature Measurement & Humidity in unsigned 16-bit int values.

- Comment: Checked the BLE documentation for the UUIDs (Temperature = 0x2AE6; Humidity = 0x2A6F) and their required data syntax type required.

Approach 4:

- Reset the Temperature characteristic UUID (Temperature Measurement = 0x2A1C) and tried to send the data as unsigned 16-bit int.

- Result: Error: Invalid Data Syntax @ nRF Application for Temperature Measurement value, but successfully received values for Humidity in unsigned 16-bit int values.

- Comment: Checked the BLE documentation for the Temperature Measurement UUID (Temperature Measurement = 0x2A1C) and their required data syntax type required.

Approach 5:

- Tried to convert the temperature data to 32-bit float and send the data along with the humidity data as unsigned 16-bit int.

- Result: Successfully received both values Temperature Measurement & Humidity in unsigned 16-bit int values, but Temperature Measurement unit was in Fahrenheit scale.

- Comment: Checked the BLE documentation for the Temperature Measurement UUID (Temperature Measurement = 0x2A1C) and their required data syntax type required and flags.

- Comment: Checked the conversion requires an IEEE11073 32-bit float data stream with 5 bytes (1 byte for C/F and 4 bytes for the actual value).

Approach 6:

- Changed the IEEE11073 32-bit float data stream with 5 bytes (1 byte for C/F and 4 bytes for the actual value) and set it to 0x00 flag for Celsius unit scale.

- Result: Successfully received both values Temperature Measurement & Humidity values on the nRF Connect Application with correct unit scales.

- Comment: Checked the BLE documentation for the Temperature Measurement UUID (Temperature Measurement = 0x2A1C) and their required data syntax type required and flags.

Outputs

Serial Monitor Messages

nRF Connect Application interface

References

- ESP32 BLE Arduino

- RuiSantosdotme

- Random Nerd Tutorials: ESP32 BLE Server Environmental Sensing Service

- Makerhero: ESP32 BLE DHT11

- ATC MiThermometer Issues

- Converting Two Bytes to IEEE 11073 16-bit sfloat in C#

- Dart: Convert IEEE 11073 32-bit Float to Simple Double

- Bosch IoT Suite Bluetooth LE Driver API

- GATT XML: Temperature Measurement

…